- Connect the Emulator Adblue to the PC via the USB cable.

- There are two diodes (blue-white and green color) light up on the Emulator.

- An unknown device appears on the PC.

- Install the driver. During the installation locate a file to the folder.

- Then open «My Computer»  «Device Manager»  «Devices»  «Com port».  Find a port where the Emulator is shown (it is called as CANBus Device in the system).

- Then, change number from the COM 5 to the COM 9 (for example) on the second line from top in the tab «Configuration».

- Open the file «configflasherflash»

- The window MS dos opens where you can see numbers with appropriate vehicle brand. For example:

0. – Daf;

5. – Volvo;

7. – Scania;

- Chose and click a number. For example if chose Volvo, click «5» and «Enter».

- Then white diode on the Emulator goes out. Only blue and green diodes light.

- After that, in a 10-15 seconds the blue diode begins to FLASHING. It means that the programming process is finished. (if blue diod light and not flashing it mean programming has been fails)

- Press «Enter».

- Take out the USB connector.

- Then you installed emulator on truck you will see blue and red lights